Single Tone

The Single Tone Definitions window allows the user to define the Duration, Frequency, Pretime, Hold Time and Minimum Decode Time parameters for the four single tones to be used in the single tone encode/decode features in the radio.

The Duration parameter is the duration of the single tone encode which may be any selected value within the specified range

The Frequency is the frequency to be allocated to each of the tones, and each of the four frequencies must be within the range 300 Hz to 3000 Hz.

Pretime is that period of time during which unmodulated carrier is transmitted before the single tone is encoded. This time must be selected from within the specified range. The purpose of this is to allow other system elements, e.g. the repeater's receive circuits, to become established.

Hold Time is the period for which the radio will remain keyed following the transmission of an encoded single tone. The period required is entered into the identified spin box and must be selected from within the specified range.

Decode Tone (Per Channel window) defines the radio's support of selective calling via single tone decode. The Decode Tone combo box allows the user to select the required single tone from the range J, K, L and M, in support of selective calling, or to disable the feature on the current channel by selecting '-' (Disabled) in the combo box.

Minimum Decode Time is the minimum period for which the tone must be received before the tone will be detected.